microct-porous-media
Scientific User Manual

Version 1.0.0

Authors

Hadi Hamaaziz Muhammed
Osnabrück University of Applied Sciences, Germany
Prof. Dr. Ruediger Anlauf
Osnabrück University of Applied Sciences, Germany

A Python package for X-ray microcomputed tomography (µCT) image analysis of soils and horticultural growing media.

Associated publication: Muhammed et al. (2025), Applied Sciences, 15(22), 11886. DOI: 10.3390/app152211886

# Table of Contents

1. Introduction

2. Scientific Background

3. Installation

4. Input Data Requirements

5. Package Structure

6. Threshold Selection

7. REV Analysis in XY Plane

8. REV Analysis in XYZ Space

9. Pore Metrics Analysis

10. Critical Pore Diameter

11. Pore Size Distribution and Water Retention Curve

12. Permeability and Saturated Hydraulic Conductivity

13. Complete Example Workflow

14. Validation Example

15. Troubleshooting

16. References and Citation

17. Appendix A: Function Reference

18. Appendix B: Output File Checklist

# 1. Introduction

microct-porous-media is a Python package for processing and analysing three-dimensional X-ray microcomputed tomography (µCT) images of soils and horticultural substrates. The package was migrated from six original research scripts and organizes them into importable modules that can be reused in scientific workflows.

The package is designed for researchers who want to extract pore-structure and hydraulic information directly from reconstructed µCT image stacks. It supports workflows from image filtering and segmentation to REV analysis, pore morphology, pore-network extraction, water-retention estimation, permeability and saturated hydraulic conductivity calculation.

## 1.1 Scientific Applications

Representative Elementary Volume (REV) analysis for visible porosity.

Quantification of pore morphology and topology.

Pore size distribution and image-based water retention curve estimation.

Critical pore diameter and percolating throat analysis.

Pore network modelling with PoreSpy and OpenPNM.

Saturated hydraulic conductivity estimation using Stokes flow and Darcy-based conversion.

## 1.2 Modules

| Module | Purpose |
| --- | --- |
| rev_xy.py | 2D REV analysis by subdividing the X-Y plane while retaining full image depth. |
| rev_xyz.py | 3D REV analysis by subdividing X, Y and Z directions simultaneously. |
| pore_metrics.py | Pore morphology, topology, percolation and network metrics. |
| critical_pore.py | Critical throat or pore diameter from a percolating network. |
| pore_size_distribution.py | Local-thickness pore size distribution and simplified water retention curve. |
| permeability.py | Pore network permeability and saturated hydraulic conductivity. |

# 2. Scientific Background

## 2.1 µCT Image Analysis

µCT provides non-destructive three-dimensional images of the pore space of soils and growing media. Reconstructed grayscale image stacks are segmented into pore and solid phases. Once binary segmentation is available, the voxel structure can be used to calculate porosity, pore morphology, topology and network-based hydraulic properties.

## 2.2 Representative Elementary Volume

The Representative Elementary Volume (REV) is the minimum volume at which a measured property becomes statistically stable. In this package, REV is evaluated using visible pore volume fraction. Stability is assessed using mean porosity, standard deviation and coefficient of variation across subvolumes.

## 2.3 Visible Porosity

Visible porosity is calculated from binary images as the number of pore voxels divided by the total number of voxels. It is resolution dependent: pores smaller than the voxel size cannot be resolved and therefore do not contribute directly to visible porosity.

## 2.4 Pore Network Modelling

Pore network modelling approximates the pore space as pores connected by throats. The package uses PoreSpy SNOW2 extraction and OpenPNM network representation. This allows pore and throat metrics as well as pressure-driven Stokes flow simulations.

## 2.5 Hydraulic Conductivity and Permeability

Permeability is an intrinsic property of a porous medium, while saturated hydraulic conductivity also depends on fluid density, viscosity and gravity. The package calculates permeability from flow rate, sample length, cross-sectional area, fluid viscosity and pressure drop. Hydraulic conductivity is then derived from permeability.

K_abs = Q * L * mu / (A * Delta_P)

Ksat = K_abs * (rho * g / mu)

# 3. Installation

## 3.1 Recommended Python Version

Use Python 3.10, 3.11 or 3.12. Python 3.14 is not recommended because several scientific dependencies such as scikit-image, PoreSpy, OpenPNM and numba may not yet provide stable binary packages for that version.

## 3.2 Python Virtual Environment

cd D:\Python Package_Image\microctpm_full_package

py -3.11 -m venv venv311

venv311\Scripts\activate

python -m pip install --upgrade pip

pip install -e .

python -c "import microctpm; print(microctpm.__version__)"

## 3.3 Conda Environment

conda env create -f environment.yml

conda activate microctpm

pip install -e .

# 4. Input Data Requirements

| Item | Requirement |
| --- | --- |
| Image format | .tif or .tiff 3D grayscale image stack. |
| Dimensions | Any 3D image shape is supported if memory is sufficient, e.g. (435,435,435), (1000,1000,1000), or non-cubic stacks such as (601,1100,1100). |
| Voxel size | Must be supplied or adjusted according to the scan resolution. Typical examples are 15 µm and 46 µm. |
| Segmentation threshold | Must be selected for each dataset. Manual, Otsu global thresholding and local thresholding are possible. |
| Memory | Large volumes can require substantial RAM, especially for PoreSpy/OpenPNM workflows. |

# 5. Threshold Selection and Segmentation

Threshold selection is one of the most important steps in the entire package. All subsequent calculations depend on whether pore and solid voxels are correctly separated.

## 5.1 Manual Thresholding

Manual thresholding uses a user-defined grayscale value. It is appropriate when the image histogram and material contrast are known. The example thresholds in the code, such as 55, 85 or 110, are dataset-specific and not universal.

binary_image = image < fixed_threshold

## 5.2 Global Otsu Thresholding

For automatic global segmentation, users may compute a threshold using Otsu method from scikit-image and pass the resulting value to the package functions.

from skimage.filters import threshold_otsu

import tifffile

image = tifffile.imread("sample.tif")

threshold = threshold_otsu(image)

print(threshold)

## 5.3 Local or Adaptive Thresholding

Local thresholding may be useful when illumination, contrast or material composition varies spatially. It is more computationally expensive and parameter dependent.

from skimage.filters import threshold_local

local_threshold = threshold_local(image_slice, block_size=51)

binary_slice = image_slice < local_threshold

## 5.4 Recommended Workflow

Inspect image slices and the grayscale histogram.

Compute an Otsu threshold as an objective reference.

Compare manual and Otsu-based segmentation visually.

Validate visible porosity against independent measurements when available.

Document the selected threshold and method in publications.

# 6. Package Structure

microctpm_full_package/

├── src/microctpm/

│   ├── rev_xy.py

│   ├── rev_xyz.py

│   ├── pore_metrics.py

│   ├── critical_pore.py

│   ├── pore_size_distribution.py

│   └── permeability.py

├── examples/

├── tests/

├── dist/

├── README.md

├── pyproject.toml

├── requirements.txt

├── environment.yml

└── LICENSE

# 7. Module: REV Analysis in XY Plane (rev_xy.py)

This module performs 2D REV analysis by dividing the X-Y plane while retaining the full Z depth. This strategy is useful for horticultural substrates where vertical gradients or packing effects may make full 3D subdivision less robust.

## 7.1 Workflow

Load 3D µCT image.

Apply Gaussian filter with sigma = 1.0.

Segment pore space using a fixed/manual threshold.

Divide the X-Y plane into n x n tiles.

Calculate visible porosity for each subvolume.

Calculate mean, standard deviation and coefficient of variation.

Export CSV files and REV plot.

## 7.2 Functions

| Function | Description |
| --- | --- |
| load_and_filter_image(image_path) | Loads a 3D TIFF image and applies Gaussian filtering. |
| create_subvolumes(image, fixed_threshold=55) | Segments the image and creates X-Y subvolumes. |
| compute_pore_volume_metrics(subvolumes, positions, voxel_size, output_dir) | Calculates porosity statistics and saves CSV/figure outputs. |
| run_rev_xy(image_path, output_dir, voxel_size=46, fixed_threshold=55) | Runs the full REV XY workflow. |

## 7.3 Parameters

| Parameter | Meaning | Typical value |
| --- | --- | --- |
| image_path | Path to 3D TIFF image. | sample.tif |
| output_dir | Folder for CSV and figure outputs. | results |
| voxel_size | Voxel size in µm. | 15 or 46 |
| fixed_threshold | Manual segmentation threshold. | Dataset-specific |

## 7.4 Division Steps

The module uses subdivision levels: 2, 3, 4, 5, 6, 7, 8, 9, 10, 15, 20, 25, 30, 35, 40, 45 and 50. For each n, n x n subvolumes are generated. The full Z depth is retained.

## 7.5 Example

from microctpm.rev_xy import run_rev_xy

results = run_rev_xy(

image_path="sample.tif",

output_dir="results",

voxel_size=15,

fixed_threshold=85

)

## 7.6 Outputs

| Output | Content |
| --- | --- |
| RHP_1_fullxy_raw_1.csv | Subvolume porosity and physical size for each tile. |
| coefficient_of_variation_RHP_1_cv.csv | Mean porosity, standard deviation and CV for each subdivision level. |
| RHP_1_xydivition.tiff | REV plot with porosity and coefficient of variation. |

# 8. Module: REV Analysis in XYZ Space (rev_xyz.py)

This module performs full 3D REV analysis by subdividing the Z, Y and X axes simultaneously. For each subdivision level n, it creates n x n x n subvolumes. This analysis is more computationally expensive than REV XY and is often better suited to mineral soils or relatively homogeneous porous media.

## 8.1 Workflow

Load and filter the 3D image.

Segment pore space using a threshold.

Divide Z, Y and X into equal segments.

Calculate visible porosity for each cube-like subvolume.

Calculate mean, standard deviation and CV.

Save CSV and TIFF outputs.

## 8.2 Functions

| Function | Description |
| --- | --- |
| load_and_filter_image(image_path) | Loads and Gaussian-filters the image. |
| create_subvolumes(image, fixed_threshold=85) | Creates 3D subvolumes after segmentation. |
| compute_pore_volume_metrics(subvolumes, positions, voxel_size, output_dir) | Computes REV metrics. |
| run_rev_xyz(image_path, output_dir, voxel_size=15, fixed_threshold=85) | Runs full 3D REV workflow. |

## 8.3 Example

from microctpm.rev_xyz import run_rev_xyz

results = run_rev_xyz(

image_path="sample.tif",

output_dir="results",

voxel_size=15,

fixed_threshold=85

)

## 8.4 Outputs

| Output | Content |
| --- | --- |
| subvolume_pore_volume_fullxyz_WF4raw_cv_1.csv | Raw subvolume porosity values and positions. |
| coefficient_of_variation_WF4raw_1.csv | CV statistics for each subdivision level. |
| pore_volume_and_cv_vs_size_full_PeatS_1_cv_3D.tiff | 3D REV plot. |

## 8.5 Interpretation

REV is indicated by stabilization of mean visible porosity and reduction of CV. REV XYZ is stricter than REV XY because heterogeneity in all three directions contributes to variability.

# 9. Module: Pore Metrics Analysis (pore_metrics.py)

pore_metrics.py is the central morphology and topology module. It calculates pore volume, pore area, pore size measures, sphericity, surface density, mean curvature density, percolation, pore class volumes, Euler number density, gamma indicator and network properties using PoreSpy and OpenPNM.

## 9.1 Main Workflow

Load image and apply Gaussian filtering.

Segment using fixed/manual threshold.

Label connected pore regions.

Calculate pore volume, pore area, equivalent diameters and sphericity.

Classify pore voxels into macro-, meso- and micropores.

Compute Euler number density and gamma indicator.

Clean binary image using remove_small_objects and remove_small_holes.

Extract pore network with PoreSpy SNOW2.

Create OpenPNM network and calculate pore/throat metrics.

Export CSV, figures and Excel summary.

## 9.2 Functions

| Function | Description |
| --- | --- |
| load_and_filter_image(image_path) | Loads 3D TIFF and applies Gaussian smoothing with sigma=2. |
| compute_euler_number(binary_image) | Calculates Euler number density from connected components and skeleton voxels. |
| compute_gamma_indicator(binary_image) | Calculates gamma indicator from connected pore cluster sizes. |
| analyze_pore_network(binary_image) | Extracts PoreSpy SNOW2 network and OpenPNM metrics. |
| analyze_pore_properties(binary_image) | Calculates pore volume, surface, diameters, sphericity and pore classes. |
| analyze_pore_structure(binary_image, original_image) | Combines morphology, topology, threshold histogram and network analysis. |
| compute_metrics(image) | Main user function that returns a pandas DataFrame and writes output files. |

## 9.3 Key Metrics

| Metric | Meaning |
| --- | --- |
| Pore Volume (µm³) | Total volume of segmented pore voxels. |
| Total Pore Area (mm²) | Estimated surface area of pore boundaries. |
| Max/Min/Avg Pore Diameter (µm) | Equivalent pore diameter from labelled pore regions or distance maps. |
| Avg Pore Sphericity | Shape descriptor indicating closeness to a sphere. |
| Pore Surface Density (mm⁻¹) | Pore surface area normalized by sample volume. |
| Mean Curvature Density (mm⁻²) | Curvature-based descriptor of pore-solid interface complexity. |
| Percolation | Whether pore space connects across the image domain. |
| Critical Pore Diameter (µm) | Limiting pore diameter computed in the morphology workflow. |
| Chi Density (mm⁻³) | Euler number density; more negative values indicate higher connectivity. |
| Gamma | Indicator of pore cluster dominance/connectivity, close to 1 for a dominant connected pore cluster. |
| Num Pores / Num Throats | Pore network size from PoreSpy/OpenPNM. |
| Avg Coordination Number | Average number of neighboring pores connected by throats. |
| Avg Throat Diameter (µm) | Mean pore throat diameter from network extraction. |
| Phi_vis (%) | Visible porosity percentage from the segmented image. |

## 9.4 Pore Classes

The module classifies pore voxels according to pore size: macropores > 300 µm, mesopores 50-300 µm and micropores < 50 µm. These thresholds are practical categories and should be interpreted relative to image resolution and research objective.

## 9.5 Example

from microctpm.pore_metrics import load_and_filter_image, compute_metrics

image = load_and_filter_image("sample.tif")

metrics = compute_metrics(image)

print(metrics)

## 9.6 Output Files

| File | Description |
| --- | --- |
| pore_properties_raw.csv | Raw pore region properties such as diameter and shape descriptors. |
| pore_sizes_raw.csv | Voxel-based pore size data. |
| pore_diameters.csv | Large pore diameter table written in chunks for memory efficiency. |
| pore_size_distribution.csv | Binned pore size distribution. |
| throat_radii.csv | Extracted network throat radii. |
| analysis_results.xlsx | Final metrics table. |
| pore_size_distribution.png | Pore size distribution plot. |
| thresholded_histogram.png | Pore and solid intensity histograms with threshold. |
| throat_size_distribution.png | Throat size distribution plot. |

## 9.7 Interpretation

High visible porosity indicates a large fraction of resolved pore space. A strongly negative Euler number density indicates many connected loops and a highly connected pore network. Gamma values close to 1 indicate that most pore voxels belong to a dominant connected cluster. High coordination number indicates many pore-to-pore connections and can be associated with increased drainage and hydraulic conductivity.

## 9.8 Warnings

FutureWarning messages from scikit-image may appear because some functions have changed names or parameters in recent versions. These warnings do not necessarily indicate failure. PoreSpy/OpenPNM warnings about unused physical properties may also appear and are not fatal unless the target metric depends on the missing property.

# 10. Module: Critical Pore Diameter (critical_pore.py)

This module estimates the critical pore or throat diameter along a percolating path through the pore network. It uses PoreSpy SNOW2 to extract a network and graph methods to evaluate inlet-to-outlet connectivity.

## 10.1 Functions

| Function | Description |
| --- | --- |
| trim_disconnected_pores(network) | Retains the largest connected pore component. |
| calculate_critical_pore_diameter(image_path, voxel_size=15e-6, threshold=85, r_max=10, sigma=1) | Runs the complete critical pore diameter workflow. |

## 10.2 Parameters

| Parameter | Meaning |
| --- | --- |
| image_path | Path to 3D TIFF image. |
| voxel_size | Voxel size in metres, e.g. 15e-6. |
| threshold | Segmentation threshold. |
| r_max | Maximum radius parameter used in local-thickness/network processing. |
| sigma | Gaussian filter sigma. |

## 10.3 Example

from microctpm.critical_pore import calculate_critical_pore_diameter

cpd = calculate_critical_pore_diameter(

image_path="sample.tif",

voxel_size=15e-6,

threshold=85,

r_max=10,

sigma=1

)

print(cpd)

## 10.4 Interpretation

The critical pore diameter represents a limiting size along a connected flow pathway. Larger critical diameters generally indicate more open percolating pathways and potentially higher drainage or conductivity. However, it should be interpreted together with porosity, connectivity and throat size distribution.

# 11. Module: Pore Size Distribution and WRC (pore_size_distribution.py)

This module estimates pore volumes, pore diameters and a simplified image-based water retention curve. It uses the local-thickness concept to infer pore size from the segmented pore space.

## 11.1 Functions

| Function | Description |
| --- | --- |
| load_and_filter_image(image_path) | Loads and filters a 3D TIFF image. |
| compute_pore_volumes_and_sizes(image) | Segments pores, applies local thickness, computes pore volumes/sizes and WRC data. |

## 11.2 Outputs

| Output | Content |
| --- | --- |
| pore_size_distribution_wrc.xlsx | Excel file containing metrics, pore data and WRC data. |
| metrics dictionary | Total pore volume, total sample volume, number of pores, average pore volume and average pore size. |
| pore_data DataFrame | Single-pore volume and equivalent pore size. |
| wrc_data DataFrame | Pore size, matric potential and volumetric water content. |

## 11.3 Example

from microctpm.pore_size_distribution import load_and_filter_image, compute_pore_volumes_and_sizes

image = load_and_filter_image("sample.tif")

metrics, pore_data, wrc_data = compute_pore_volumes_and_sizes(image)

print(metrics)

print(wrc_data.head())

## 11.4 Interpretation

Large pores drain at lower matric suction, while smaller pores retain water at higher suction. The image-based WRC is resolution-limited because pores smaller than the voxel size cannot be resolved. Users should compare the resulting WRC with laboratory measurements whenever possible.

# 12. Module: Permeability and Saturated Hydraulic Conductivity (permeability.py)

This module estimates intrinsic permeability and saturated hydraulic conductivity by extracting a pore network, assigning hydraulic conductance and solving Stokes flow using OpenPNM.

## 12.1 Workflow

Load and Gaussian-filter image.

Segment pores by threshold.

Optionally save image histogram and binary slice.

Pad image for SNOW2 extraction.

Extract network with PoreSpy.

Convert to OpenPNM network.

Trim disconnected components.

Scale geometry by voxel size.

Add geometry and hydraulic conductance models.

Define inlet and outlet pores.

Solve Stokes flow.

Compute permeability and hydraulic conductivity.

Optionally save 3D network/pressure figure.

## 12.2 Function

| Function | Description |
| --- | --- |
| calculate_permeability(...) | Main workflow for permeability and Ksat calculation. |
| calculate_hydraulic_conductivity(...) | Alias for calculate_permeability. |
| trim_disconnected_pores(network) | Keeps the largest connected network component. |

## 12.3 Important Parameters

| Parameter | Description | Example |
| --- | --- | --- |
| image_path | Path to 3D TIFF image. | sample.tif |
| output_dir | Output folder. | results |
| voxel_size | Voxel size in metres. | 15e-6 |
| threshold | Segmentation threshold. | 85 |
| sigma | Gaussian filter sigma. | 1 |
| boundary_width | SNOW2 boundary width. | 5 |
| inlet_pressure | Inlet pressure in Pa. | 2000 |
| outlet_pressure | Outlet pressure in Pa. | 1990 |
| pressure_drop | Pressure drop used in Darcy-law permeability calculation. | 1990 |
| viscosity | Dynamic viscosity of water in Pa s. | 1.0e-3 |
| density | Water density in kg m-3. | 1000 |
| gravity | Gravitational acceleration in m s-2. | 9.81 |
| flow_axis | Flow axis: 0, 1 or 2. | 0 |

## 12.4 Pressure Drop

The pressure boundary conditions used by the flow solver and the pressure drop used in Darcy-law conversion should be handled carefully. In your validated workflow, pressure_drop=1990 Pa reproduced the original script convention and produced realistic hydraulic conductivity for the tested image. If pressure_drop is omitted, the package may use abs(inlet_pressure - outlet_pressure), depending on the implementation. Users must document the selected pressure-drop convention.

Pressure Boundary Conditions and Pressure Drop

The permeability calculation is based on a pressure gradient imposed across the extracted pore network.

Users may specify:

inlet_pressure

outlet_pressure

pressure_drop

The pressure boundary conditions define the hydraulic gradient that drives flow through the pore network.

If:

pressure_drop = None

the package automatically calculates:

ΔP = |P_inlet − P_outlet|

Example:

inlet_pressure = 2000 Pa

outlet_pressure = 1990 Pa

gives:

ΔP = 10 Pa

Important Scientific Note

Pressure drop is not a universal constant.

The appropriate pressure difference depends on:

pore size distribution

throat size distribution

network connectivity

hydraulic gradient

experimental conditions

imposed boundary conditions

Therefore, a pressure difference used for one porous medium should not automatically be applied to another porous medium.

The values shown in examples are intended only to demonstrate package usage and should not be interpreted as universal settings.

Recommended Practice

For general use:

pressure_drop = None

For research applications:

Use pressure boundary conditions that correspond to the experimental or modelling scenario being investigated.

Always report:

inlet_pressure

outlet_pressure

pressure_drop

when publishing permeability or hydraulic conductivity results.

12.5 Example

## 12.5 Example

from microctpm.permeability import calculate_permeability

results = calculate_permeability(
image_path="sample.tif",
output_dir="results",
voxel_size=15e-6,
threshold=85,
inlet_pressure=2000.0,
outlet_pressure=1990.0,
pressure_drop=None,
save_outputs=True,
make_plot=True,
show_plot=False
)

print(results)

## 12.6 Outputs

| Output | Meaning |
| --- | --- |
| porosity | Visible porosity of segmented image. |
| flow_rate_m3_s | Simulated flow rate. |
| permeability_m2 | Intrinsic permeability in square metres. |
| permeability_mD | Permeability in millidarcy. |
| hydraulic_conductivity_cm_s | Saturated hydraulic conductivity in cm/s. |
| num_pores / num_throats | OpenPNM network size. |
| num_inlet_pores / num_outlet_pores | Boundary pores used by Stokes flow. |
| pressure_drop_Pa | Pressure drop used in Darcy-law calculation. |
| intensity_histogram.tif | Optional grayscale histogram. |
| binary_image_slice.tif | Optional segmented slice. |
| 3d_pore_network.tif | Optional network/pressure visualization. |

# 13. Complete Example Workflow

The following workflow demonstrates how a user can analyse a 3D µCT image named sample.tif. Replace voxel size and threshold with values appropriate for the dataset.

# 1. REV XY

from microctpm.rev_xy import run_rev_xy

run_rev_xy("sample.tif", "results/rev_xy", voxel_size=15, fixed_threshold=85)

# 2. REV XYZ

from microctpm.rev_xyz import run_rev_xyz

run_rev_xyz("sample.tif", "results/rev_xyz", voxel_size=15, fixed_threshold=85)

# 3. Pore metrics

from microctpm.pore_metrics import load_and_filter_image, compute_metrics

image = load_and_filter_image("sample.tif")

metrics = compute_metrics(image)

# 4. Critical pore diameter

from microctpm.critical_pore import calculate_critical_pore_diameter

cpd = calculate_critical_pore_diameter("sample.tif", voxel_size=15e-6, threshold=85)

# 5. Pore size distribution and WRC

from microctpm.pore_size_distribution import load_and_filter_image, compute_pore_volumes_and_sizes

image = load_and_filter_image("sample.tif")

metrics, pore_data, wrc_data = compute_pore_volumes_and_sizes(image)

# 6. Permeability and Ksat

from microctpm.permeability import calculate_permeability

k = calculate_permeability("sample.tif", output_dir="results/permeability", voxel_size=15e-6, threshold=85, pressure_drop=1990.0)

# 14. Validation Example

During package testing, a real cropped 3D µCT image was processed through all six modules. The exact values are dataset-specific and should be treated as validation evidence for the tested image, not universal expected results.

| Metric | Validated example value |
| --- | --- |
| Image size | 435 x 435 x 435 voxels |
| Voxel size | 15 µm |
| Threshold | 85 |
| Visible porosity from pore metrics | 64.30 % |
| Critical pore diameter | 222.62 µm |
| Pore network size | 15,649 pores and 55,685 throats in pore_metrics workflow |
| Permeability | 3.16e-11 m² |
| Permeability | 32,002 mD |
| Hydraulic conductivity | 0.0310 cm/s |
| Pressure drop used | 1990 Pa |

# 15. Troubleshooting

| Problem | Likely cause | Solution |
| --- | --- | --- |
| ModuleNotFoundError: microctpm | Package not installed or wrong environment. | Activate the correct environment and run pip install -e . |
| Python 3.14 installation failure | Scientific dependencies unavailable for very new Python version. | Use Python 3.10-3.12, preferably Python 3.11. |
| FileNotFoundError for output folder | Output directory does not exist. | Create output folder or use a function that creates it automatically. |
| local_thickness unexpected keyword mode | PoreSpy API version mismatch. | Remove unsupported mode argument or use compatible PoreSpy version. |
| Tkinter or FigureCanvasAgg warning | Non-interactive plotting backend. | Use show_plot=False and save figures to files. |
| OpenPNM warnings about surface_tension or diffusivity | Models not needed for hydraulic conductance were skipped. | Usually safe to ignore unless those properties are required. |
| Very slow SNOW2 extraction | Image too large or high porosity network. | Crop image, increase RAM, or process REV-sized subvolume. |
| Unrealistically high Ksat | Pressure drop convention or segmentation threshold may be wrong. | Check pressure_drop, inlet/outlet pressures, threshold and connectivity. |

# 16. References and Citation

Primary scientific reference for the methodology:

Muhammed, H. H.; Anlauf, R.; Daum, D.; Schmitt Rahner, M.; Kuka, K. (2025). Pore Structure Analysis of Growing Media Using X-Ray Microcomputed Tomography. Applied Sciences, 15(22), 11886. DOI: 10.3390/app152211886

## 16.1 Suggested Package Citation

@software{microct_porous_media_2026,

author = {Muhammed, Hadi Hamaaziz and Anlauf, Ruediger},

title = {microct-porous-media: Python tools for µCT image analysis of soils and horticultural substrates},

version = {1.0.0},

year = {2026}

}

## 16.2 Paper BibTeX

@article{Muhammed2025MicroCT,

author = {Muhammed, Hadi Hamaaziz and Anlauf, Ruediger and Daum, Diemo and Schmitt Rahner, Mayka and Kuka, Katrin},

title = {Pore Structure Analysis of Growing Media Using X-Ray Microcomputed Tomography},

journal = {Applied Sciences},

year = {2025},

volume = {15},

number = {22},

pages = {11886},

doi = {10.3390/app152211886}

}

# Appendix A: Function Reference

| Module | Public functions |
| --- | --- |
| rev_xy.py | load_and_filter_image; create_subvolumes; compute_pore_volume_metrics; run_rev_xy |
| rev_xyz.py | load_and_filter_image; create_subvolumes; compute_pore_volume_metrics; run_rev_xyz |
| pore_metrics.py | load_and_filter_image; compute_euler_number; compute_gamma_indicator; analyze_pore_network; analyze_pore_properties; analyze_pore_structure; compute_metrics |
| critical_pore.py | trim_disconnected_pores; calculate_critical_pore_diameter |
| pore_size_distribution.py | load_and_filter_image; compute_pore_volumes_and_sizes |
| permeability.py | trim_disconnected_pores; calculate_permeability; calculate_hydraulic_conductivity |

# Appendix B: Output File Checklist

Always check CSV and Excel files after each run.

Compare visible porosity with expected or laboratory porosity when available.

Inspect saved plots for axis labels, threshold quality and unexpected distributions.

Record threshold, voxel size, image dimensions and preprocessing settings for reproducibility.

For permeability, always report pressure boundary conditions and pressure_drop used in Darcy conversion.